ESS422					Name:_________________________

This lab is due on Thursday, April 9th.

LAB 1:  Shade

In this lab you will examine shade in an image using MASTER data from Gifford Pinchot National Forest, WA.

The data are located on a server.  Log onto the classroom computer using "geo-user", password "temporary".  There is a shortcut icon on the desktop called "shortcut to geo-user".  Go to the ESS422 folder.  There are four folders with images that will be used for the labs.  Download the image you will be working on to the local "D" drive in a folder with your name.

A.  Open Giff_Pinch_Master.HDF.  Examine the darkest areas in the image.

1.  How many different types of dark objects can you locate?  Record the type of dark object, their

locations and DN values.

2.  Make ROI's of the dark areas and examine their spectra.  Look at different bands for Rayleigh

Scattering.  How do the dark objects behave?

3.  Do trees show Lambertian behavior?  How is a shadow cast by a tree different than one cast by

a mountain?

4.  Dark object subtraction is often used to correct for atmospheric affects.  Which dark area would

you choose in this image for a dark object subtraction correction and why?