ESS422					Name:_________________________

This lab is due Thursday, April 23rd.

LAB 3:  Classification

In this lab you will examine different classifications, apply an atmospheric correction and try channel ratios.  The image is the ASTER image of Yuma, Arizona with channels 1, 2, and 3 (VNIR: Green, Red, Near IR) re-sampled  to match channels 4-9 (SWIR) in size and pixel resolution.

A.  Open Yuma_ASTER.HDF

1.  Choose several types of landscapes (e.g., forest, water, etc.) from the scene using the ROI tool.

Run four classifications:  K-Means, Parallelepiped, Maximum Likelihood and one of your

choice.

On the main menu click on Classification—> Supervised

Choose Parallelepiped or Maximum Likelihood

In the Parameters Box choose 'Select All Items' for your regions

Output result to Memory or choose an output file name

Change 'Output rule image' to 'No'

Click OK

Feel free to experiment with the Standard Deviation Parameters

K-Means will be in Classification—> Unsupervised

a.  Compare the results of the four different classifications.  Note your observations.

b.  Which classification do you think was the most successful, and why?

B.  Apply a 'dark object subtraction' atmospheric correction to the image.  This is the Path Radiance

correction.

1.  Find a hill in the image that looks to be the same except for lighting geometry.  Record the

DN's for channels 1 and 3. and their ratio value for the sunny and shaded side.  It the ratio the

same or different on sunlit and shaded sides of the same hill?

2.  Find the DN value of the darkest pixel in the image.  Make an ROI of a group of pixels

surrounding the darkest pixel (to be sure it's not an artifact or noise).  Record the approximate

location of the region.

3.  Use 'Dark Subtract' for the correction:

Go to Basic Tools —> General Purpose Utilities—> Dark Subtract

Click on the Yuma file, then OK

Choose Region of Interest as the Subtraction Method and select the ROI

Choose an output file name.

4.  Remake the ratio of Step 1 now that you have corrected for atmospheric path radiance.  What

are the ratio values now?

5.  Sketch the histogram for channel 1 before and after the correction and note any changes.

C.  Apply a transmissivity correction to the image using MODTRAN.

1.  Use the PPT tutorial to set up and run MODTRAN.

2.  Once you have the wavelength in microns, average the transmissivity values over the

nine Aster channel ranges.  Record the averages below.  An average is being used to

simplify this exercise.

Aster Channel Wavelength Ranges		     Average Transmissivity

Channel 1:  0.52-0.60			___________________________

Channel 2:  0.63-0.69			___________________________

Channel 3:  0.76-0.86			___________________________

Channel 4:  1.6-1.7			___________________________

Channel 5:  2.145-2.185			___________________________

Channel 6:  2.185-2.225			___________________________

Channel 7:  2.235-2.285			___________________________

Channel 8:  2.295-2.365			___________________________

Channel 9:  2.360-2.430			___________________________

3.  Apply the average Transmissivity correction to the image using 'Apply Gaim and Offset.'

Go to Basic Tools—>General Purpose Utilities—>Apply Gain and Offset

Choose the file that has been 'dark object' corrected, click OK

Under 'Gain Values' click on each band (channel) and enter the corresponding

transmissivity value.  Offset values should be zero since the dark object subtraction

has already been made.  Both steps can be performed together using this utility if you

do 'stats' on the dark ROI, then record the average in the 'Offset Values' option.

This will give the final atmospheric correction.  View the images and their histograms and note

your observations.

4.  If you were to remake the ratios from B again now that you have corrected for transmissivity,

would the new values be the same or different from these in B 4?

D.  Band Ratios minimize surface roughness effects at all scales.

1.  Using Band Math or Transform—>Ratios, make three ratios of the atmospherically corrected

VNIR channels (1/2, 1/3, 2/3).  Are the ratios useful in reducing topographic effects?